Stage 2 Plan – Rhys Cornelious

The radio signal will be captured using the RTL-DTR Blog V4, a kit which includes an antenna receiver and SMA dongle to transmit data via USB. This allows it to interface with the Pi and provide the script with the audio information required. I believe that a Pi V5 with 4GB of RAM is a safe amount to run the audio to MIDI script and have more than enough bandwidth for all other processes including the SDR reception. Furthermore, a 32GB SD card will be more than sufficient, and the A1 classification means it should not have issues reading and writing from memory at the rates I expect we will need.

In order to play the audio, we require speakers. The Logitech S150 set is powered by USB and receives its input data through the same source, thus making them a good low-cost solution that can interface easily with the Pi. The other accessories that we need, before moving into components for the UI, are a power supply for the Pi as well as a cooling fan to ensure that it can continuously stream radio audio over extended periods without overheating.

The user interface will grant controls for seeking up and down to the next available channel, volume control, power control, and output audio stream control. In order to change channels, normally open buttons will work effectively and be intuitive to use. Another of these buttons can be implemented to turn the radio on and off easily, preventing the user from needing to unplug the system each time. The rotary encoder will provide volume control, while an on-off-on switch will allow the user to toggle between direct radio streaming, MIDI-only output, and a combined stream of both. Finally, an I2C compatible LCD screen should provide an easy way to display the current channel and/or volume level while giving the system an old-school radio look. The breadboard and wiring kits will be used to connect all of the components, and I have soldering equipment available if needed to ensure more secure connections. All of the components user for interaction were selected to be panel mounted, and I will be taking advantage of some nearby resources to 3D print a simple enclosure that will allow me to mount and label all the controls.

All components were found available on Amazon, DFRobot, and Digikey – reputable vendors with reasonable pricing. The total BOM comes out to 214.63 before tax, which is estimated at 235.56 after tax based on local rates.